Safety Data Sheet

The Wipe Company Ltd

IMPREG 26 Quat Free Udder wipe  CLP

Issue 2 June 2018

SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifier

| Product Name | The Wipe Company IMPREG 26 CLP |
| --- | --- |

  - Relevant identified uses of the substance or mixture and uses advised against

| Product Use | [SU22] Professional uses: Public domain (administration, education, entertainment, services, craftsmen); [SU10] Formulation [mixing] of preparations and/or re-packaging (excluding alloys); [PC8] Biocidal products (e.g. Disinfectants, pest control); [PROC8b] Transfer of substance or preparation (charging/discharging) from/to vessels/large containers at dedicated facilities; [ERC5] Industrial use resulting in inclusion into or onto a matrix; [AC8] Paper articles; |
| --- | --- |

  - Details of the supplier of the safety data sheet

| Company Address | The Wipe Company Ltd 3-5 Millen Road Sittingbourne Kent ME10 2BQ United Kingdom www.thewipecompany.co.uk Tel: 01795 410202 Fax: 01795 420406 Email: steve@thewipecompany.co.uk |
| --- | --- |

  - Emergency telephone number

| Emergency Telephone Number | The Wipe Company Ltd 01795 410202 |
| --- | --- |

SECTION 2: Hazards identification

2.1 Classification of the substance or mixture

| 2.1.1. Classification- 67/548/EEC Main Hazards 2.1.2. Classification – EC 1272/2008 | R52/53 Symbols: Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment. EUH208; Aquatic Chronic 3: H412; |
| --- | --- |

2.2 Label elements

| Hazard Statement | EUH208 - Contains polyaminopropyl-biguanide. May produce an allergic reaction. Aquatic Chronic 3: H412 - Harmful to aquatic life with long lasting effects. |
| --- | --- |
| Precautionary Statement: Prevention | P102 - Keep out of reach of children. P103 - Read label before use. P273 - Avoid release to the environment |
| Precautionary Statement: Disposal | P101 - If medical advice is needed, have product container or label at hand. P301+P312 - IF SWALLOWED: Call a POISON CENTER/doctor/ /if you feel unwell. P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| Precautionary Statement: Disposal | P501 - Dispose of contents/container to a licensed waste disposal company in compliance with local and national regulations. |

2.3 Other hazards

| Other hazards | This product does not contain any PBT or VPvB substances. |
| --- | --- |

SECTION 3: Composition/information on ingredients

3.1 Substances

67/548/EEC/ 1999/45/EC

| Chemical Name | Index No. | CAS No. | EC No. | REACH Reg No. | Conc. (%w/w) | Classification |
| --- | --- | --- | --- | --- | --- | --- |
| Ethanol | 603-002-00-5 | 64-17-5 | 200-578-6 |  | 1-10% | F; R11 |

EC 1272/2008

| Chemical Name | Index No. | CAS No. | EC No. | REACH Reg No. | Conc. (%w/w) | Classification |
| --- | --- | --- | --- | --- | --- | --- |
| Ethanol | 603-002-00-5 | 64-17-5 | 200-578-6 |  | 1-10% | Flam Liq.2: H225; |
| GLYCEROL (Glycerol, mist) |  | 56-81-5 | 200-289-5 |  | 1 - 10% |  |
| POLYMERIC BIGUANIDE HYDROCHLORIDE |  | 27083-27-8 |  |  | 0 - 0.5% | Acute Tox. 4: H302; Skin Sens. 1: H317; Eye Dam. 1: H318; Carc. 2: H351; STOT RE 1: H372; Aquatic Acute 1: H400; Aquatic Chronic 1: H410; |

3.2 Mixtures

|  | See section 3.1 for further information |
| --- | --- |

Further information

|  | For full text of H-statements and R-phrases: see SECTION 16. |
| --- | --- |

SECTION 4: First Aid Measures

4.1 Description of first aid measures

| Inhalation Eye contact Skin contact Ingestion | Considered to be not hazardous by inhalation. Seek medical attention if irritation or symptoms persist. Rinse immediately with plenty of water for 15 minutes holding the eyelids open. Seek medical attention ir irritation or symptoms persist. No irritation expected. Seek medical attention it irritation or symptoms persist. Rinse mouth thoroughly. DO NOT INDUCE VOMITING. Seek medical attention it irritation or symptoms persist. |
| --- | --- |

4.2 Most important symptoms and effects, both acute and delayed

| Inhalation Eye contact Skin contact Ingestion | No specific symptoms noted. May cause irritation to eyes. May cause allergic reactions in susceptible people. No specific symptoms noted. |
| --- | --- |

4.3 Indication of any immediate medical attention and special treatment needed

|  | If you feel unwell, seek medical advice (show label where possible). No specific first aid measures noted. |
| --- | --- |

SECTION 5: Firefighting measures

5.1 Extinguishing media

|  | This product is not flammable. Use extinguishing media appropriate to the surrounding fire conditions. |
| --- | --- |

5.2 Special hazards arising from the substance or mixture

|  | None. |
| --- | --- |

5.3 Advice for firefighters

|  | No specific firefighting procedure given. |
| --- | --- |

SECTION 6: Accidental release measures

6.1 Personal precautions, protective equipment and emergency procedures

|  | Avoid contact with eyes. Surfaces contaminated with the product will become slippery. Wear suitable protective clothing. |
| --- | --- |

6.2 Environmental precautions

|  | Product cannot be spilled |
| --- | --- |

6.3 Methods and material for containment and cleaning up

|  | Product cannot be spilled |
| --- | --- |

6.4 Reference to other sections

|  | For waste disposal see Section 13. For personal protection see section 8. |
| --- | --- |

SECTION 7: Handling and storage

7.1 Precautions for safe handling

|  | Avoid contact with eyes. Do not eat, drink or smoke in areas where this product is used or stored. Wash hands after handling the product. Remove contaminated clothing and protective equipment before entering eating areas. Adopt best Manual Handling considerations when handling, carrying and dispensing. |
| --- | --- |

7.2 Conditions for safe storage, including any incompatibilities

|  | Keep out of the reach of children. Keep in a cool, dry, well ventilated area. Keep containers tightly closed. Store in correctly labelled containers. Do NOT allow to freeze. |
| --- | --- |

7.3 Specific end use(s)

|  | Restricted to professional users. |
| --- | --- |

SECTION 8: Exposure controls/personal protection

8.1 Control parameters

8.1.1. Exposure Limit Values

| Ethanol | WEL 8-hr limit ppm: WEL 15 min limit ppm: WEL 8-hr limit mg/m3 total inhalable dust: WEL 8-hr limit mg/m3 total respirable dust | 1000 WEL 8-hr limit mg/m WEL 15 min limit mg/m33: WEL 15 min limit mg/m3 total inhalable dust WEL 15 min limit mg/m3 total respirable dust: | 1920 |
| --- | --- | --- | --- |

8.2 Exposure Controls

| 8.2.1 Appropriate engineering controls 8.2.2. Individual Protection measures Eye/face protection 8.2.3. Environmental exposure controls | Keep in a cool, dry, well ventilated area. Wear protective clothing. In case of splashing, wear: Approved safety goggles. Provide eye wash station. Do not empty into drains. Do not flush into surface water. Prevent further spillage if safe. |
| --- | --- |

Further information

|  | Information valid at the time of review of safety data sheet. |
| --- | --- |

SECTION 9: Physical and chemical properties

9.1 Information on basic physical and chemical properties

| State Colour Odour Odour threshold pH Melting point Freezing point Boiling point Flash point Evaporation rate Flammability limits Vapour flammability Upper Explosive Limit Lower Explosive Limit Vapour pressure Vapour density Relative density Partition coefficient (n-octano/water) Autoignition temperature Viscosity Explosive Oxidising Solubility | Wet Cloth Colourless Slight Not determined 5 - 6 Not determined Not determined Not determined Not determined Not determined Not determined Not determined Not determined Not determined Not determined Not determined 0.98 – 0.99 Not determined Not determined Not determined Not determined Not determined Soluble in water |
| --- | --- |

9.2 Other information

|  | Not relevant. |
| --- | --- |

Further information

| Supports combustion? | This product does not support combustion. |
| --- | --- |

SECTION 10: Stability and reactivity

10.1 Reactivity

|  | No specific reactivity hazards associated with this product |
| --- | --- |

10.2 Chemical stability

|  | Stable under normal conditions. This product is not flammable. Will not decompose if stored and used as recommended. |
| --- | --- |

10.3 Possibility of hazardous reactions

|  | Strong acids and strong bases. Strong oxidising agents. |
| --- | --- |

10.4 Conditions to avoid

|  | Heat. Do NOT allow to freeze. |
| --- | --- |

10.5 Incompatible materials

|  | Strong acids. Strong bases. Strong oxidising agents. |
| --- | --- |

10.6 Hazardous decomposition products

|  | Burning produces irritating, toxic and obnoxious fumes. |
| --- | --- |

SECTION 11: Toxicological information

11.1 Information on toxicological effects

| Acute toxicity | Based on available data the classification criteria are not met. |
| --- | --- |
| Skin corrosion/irritation | Based on available data the classification criteria are not met. |
| Serious eye damage/irritation | May cause irritation to eyes |
| Respiratory or skin sensitisation | May cause allergic reactions in susceptible people |
| Germ cell mutagenicity | No mutagenic effects reported. |
| Carcinogenicity | No carcinogenic effects reported. |
| Reproductive toxicity | No teratogenic effects reported. |
| STOT-single exposure | Not classified as a specific target organ toxicant after a single exposure |
| STOT-repeated exposure | Not classified as a specific target organ toxicant after repeated exposure. |
| Aspiration hazard | Based on available data the classification criteria are not met. |

SECTION 12: Ecological Information

12.2 Persistence and degradability

|  | The surfactant(s) contained in this preparation complied (comply) with the biodegradability criteria as laid down in the Regulation (EC) No.648/2004 on detergents. Data to support this assertion are held at the disposal of the competent authorities of the Member States and will be made available to them, at their direct request or at the request of a detergent manufacturer. |
| --- | --- |

12.3 Bioaccumulative potential

|  | No data is available on this product. |
| --- | --- |

12.4 Mobility in soil

|  | The product is soluble in water. |
| --- | --- |

12.5 Results of PBT and vPvB assessment

|  | This product does not contain any PBT or vPvB substances. |
| --- | --- |

12.6 Other adverse effects

|  | Not Known. |
| --- | --- |

SECTION 13: Disposal considerations

13.1 Waste treatment methods

|  | Contact a licensed waste disposal company. Dispose of as special waste in compliance with local and national regulations. For disposal within the EC, the appropriate code according to the European Waste Catalogue (EWC) should be used. Empty containers can be sent to landfill after cleaning, if in compliance, with local and national regulations. |
| --- | --- |

SECTION 14: Transport Information

14.6 Special precautions for user

|  | Not applicable |
| --- | --- |

14.7 Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

|  | Not Applicable |
| --- | --- |

ADR/RID

|  | The product is not classified as dangerous for carriage |
| --- | --- |

IMDG

|  | The product is not classified as dangerous for carriage |
| --- | --- |

IATA

|  | The product is not classified as dangerous for carriage |
| --- | --- |

Further information

|  | The product is not classified as dangerous for carriage |
| --- | --- |

SECTION 15: Regulatory Information

15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture

|  | Commission Directive 2000/39/EC of 8 June 2000 established a first list of indicative occupational exposure limit values in implementation of Council Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents at work. Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Regulation, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93 and Commission Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC, including amendments. Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006 with amendments. Commission Directive 91322/EEC of 29 May 1991 on establishing indicative limit values by implementing Council Directive 80/1107/EEC on the protection of workers from the risks related to exposure to chemical, physical and biological agents at work |
| --- | --- |

15.2 Chemical safety assessment

|  | No chemical safety assessment has been carried out. |
| --- | --- |

SECTION 16: Other information

Other information

| Text of Hazard Statements in Section 3 | Flam. Liq. 2: H225 - Highly flammable liquid and vapour. Acute Tox. 4: H302 - Harmful if swallowed. Skin Sens. 1: H317 - May cause an allergic skin reaction. Eye Dam. 1: H318 - Causes serious eye damage. Carc. 2: H351 - Suspected of causing cancer . STOT RE 1: H372 - Causes damage to organs through prolonged or repeated exposure . Aquatic Acute 1: H400 - Very toxic to aquatic life. Aquatic Chronic 1: H410 - Very toxic to aquatic life with long lasting effects. |
| --- | --- |
| Glossary: | ADR European Agreement concerning the International Carriage of Dangerous Goods by Road IATA International Air Transport Association IMDG International Maritime Dangerous Goods PBT Persistent, bioaccumulative, toxic RID Convention concerning International Carriage by Rail vPvB very persistent, very bioaccumulative STOT Specific Target Organ Toxicity WEL Workplace Exposure Limit |

General Information

|  | Only trained personnel should use this material. |
| --- | --- |

Further information

|  | The information supplied in this Safety Data Sheet is designed only as guidance for the safe use, storage and handling of the product. This information is correct to the best of our knowledge and belief at the date of publication however no guarantee is made to its accuracy, reliability or completeness. This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any other process. It is the user’s responsibility to satisfy him/herself as to the suitability of such information for his/her own particular use. |
| --- | --- |